Project Name:

SECTION 12492

Horizontal Louver Blinds

# Part 1 - General

# DESCRIPTION

- 1" Horizontal Wood Blinds

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Submit working hand sample or mock up blind as required.

- Submit two 6" samples of wood slat indicating color and dimensions.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions LLC or architect approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - HORIZONTAL BLINDS

- Product: Graber® Traditions™ 1” Wood Blinds

- Color Name:      Color Number:

- Headrail shall be 1" high x 1 1/2" wide x .025” thick U-shaped steel. The steel finishing process includes phosphate treatment for corrosion resistance, a chrome-free sealer, a low HAP urethane primer and a topcoat with low HAP polyester baked enamel.

- Tilter shall be made of injection-molded thermoplastics for smooth low-friction operation and will incorporate a clutch mechanism to prevent damage due to over tilting.

Select One:

Wood wand tilter (standard)

Ring tilter

- Cord lock shall be a snap-in design incorporating a stainless steel wear guard over which cords pass. A floating, shaft-locking pin will lock blind at any position and shall be crash proof. End of lift cords shall be treated with wooden tassels.

Options:

Ring pull provides a single plated steel ring in lieu of tassels with a nominal 4" cord length.

- Braided ladder shall be made of 100% polyester incorporating two extra strength rungs per ladder to support slats.

- Slats shall be 1” wide and made from North American hardwood and are resistant to warping.

- Bottomrail shall be a 5/8” high x 1” wide wood and finished to coordinate with slat color.

  - Fabrication

- Blinds shall be fabricated according to specifications and accurate to tolerance established by SWF engineering standards

# Part 3 – Execution

  - Inspection

- installer shall be responsible for inspection of jobsite, approval of mounting surfaces, verification of field measurements and installation conditions.  Installation shall commence when satisfactory conditions are met.

  - Installation

- Install blinds in accordance with manufacturer’s instructions including recommended support brackets and fasteners.

- Install blinds with adequate clearance to permit smooth operation of the blinds.  Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

- Maintain and clean blinds in accordance with manufacturer’s instructions.

### End of Section