Project Name:

SECTION 12492

Vertical Louver Blinds

# Part 1 - General

# DESCRIPTION

- Vertical Louver Blinds

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Submit working hand sample or mock up blind as required.

- Submit two 6" samples of louvers indicating color.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions LLC or architect approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - VERTICAL BLINDS

- Product: Graber® G-85 Dura-Vue™ Vertical Blinds

- Headrail shall be roll formed, phosphate-treated recycled steel with a with baked enamel finish in Ivory Gloss. Headrail shall measure 1 1/2" wide x 1 1/4" high x .027” thick.

- Carriers shall be molded nylon on traversing wheels and centrally located making the headrail reversible.  Detachable stems shall be clear, non-yellowing, UV-stabilized nylon and are impact resistant. Carriers and stems shall be replaceable without demounting the headrail.

- Rotation controls shall consist of #6 nickel-plated steel bead chain and a rack and pinion gear system providing 180º direct rotation. The pinion rod shall be extruded aluminum 0.30” diameter.

Select One:

#6 Nickel Plated Steel Bead Chain (standard)

One Touch wand control 
 	     (one way draw only, not available with 2” louvers)

- Traversing control shall be a nylon strap system. Traverse cord is #3 braided polyester with a fiberglass core, 0.094” diameter and is equipped with a cord weight and cord clip. The cord clip is anchored to the wall or window jamb during installation in accordance with CPSC recommendations.

- Louvers shall be selected from the Graber® line of PVC offerings in 2” or 3½” widths and/or fire-rated fabrics in 3½” widths.  FR certifications are available upon request.

Select One

3 ½” PVC      Color Number       Color Name

2” PVC      Color Number       Color Name

3 ½” Fabric      Color Number       Color Name

Options

Channel Panel Insert for Fabric Louvers

Color Name:      Color Number:

Bottom chain (N/A on channel panel or 2” PVC louvers)

  - Fabrication

- Blinds shall be fabricated according to specifications and accurate to tolerance established by SWF engineering standards

# Part 3 – Execution

  - Inspection

- installer shall be responsible for inspection of jobsite, approval of mounting surfaces, verification of field measurements and installation conditions.  Installation shall commence when satisfactory conditions are met.

  - Installation

- Install blinds in accordance with manufacturer’s instructions including recommended support brackets and fasteners.

- Install blinds with adequate clearance to permit smooth operation of the blinds.  Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

- Maintain and clean blinds in accordance with manufacturer’s instructions.

### End of Section